Title: Molecularly Imprinted Polymers based on Conductive Polymers: Useful Materials for Chemical Sensing.

Name: M. Debliquy*1, Hugues Charlier1, D. Lahem2, X. Tang3, A. Gonzalez Vila1, J.-P. Raskin3, C. Zhang4, C. Caucheteur1

*corresponding author

Email: marc.debliquy@umons.ac.be

1. Materials Science Department, Université de Mons, 7000 Mons Belgium

2. Department of Materials Science, Materia Nova ASBL, 7000 Mons, Belgium

3. ICTEAM, Université catholique de Louvain, 1348 Louvain-la-Neuve, Belgium

4. College of Mechanical Engineering, Yangzhou University, Yangzhou 225127, China

These last few decades, the demand for chemical detection methods and measurement techniques has been constantly increasing. This demand is driven by environmental, security and process control concerns and by the need for efficient and quick medical diagnostics. Classical methods based on sample collection and lab measurements are the most used measurement techniques. They are accurate and

reliable but they are expensive, not real time and cause statistics issues. Devices based on chemical sensors are a good solution as these systems can be very sensitive, low cost, small size, real time and easily integrated in common electronics to build portable systems. This paper presents chemical sensors based on molecularly imprinted polymer (MIP) built with conductive polymers as polymer matrix.

MIP is a polymer synthesized by incorporating a template molecule. Functional monomers form a complex around the template and are linked afterwards to form a polymer constituted of a series of “cages” trapping the template. Afterwards, the template is removed, leaving cavities imprinted in the polymer matrix that allows the polymer to selectively recognize the target molecule. These elements are cheap, easy to synthesize and can be adapted to any kind of surface. When using conducting polymers for building MIP’s, the adsorption of the template can modify their optical or electrical properties. We showed that it was possible to use these materials for building chemoresistive, potentiometric or optical fiber sensors for the detection of pollutants in air, glucose or molecules of medical interest with a good selectivity and sensitivity.

Biography

Marc Debliquy received his Ph.D. at the Faculty of Engineering in Mons (Belgium) in 1999. He joined the company Sochinor in 2000 and then Materia Nova in 2003. Since October 2008, he joined the Material Science Department of University of Mons and worked as a team leader of the sensor group. He was promoted associate professor in September 2013. His main research interest is smart coatings for chemical detection. He is (co)author of more than 55 articles in international journals, 10 book chapters and 60 proceedings. He is also co-founder of the company B-Sens active in the field of sensors.

Photograph: