Development of an MRI-Based Technique for the Non-Invasive Diagnosis of Adenomyosis in a

Murine Model

J. Vervier1,2, M. Squatrito2, D. Danthine3, S. Boutry4, M. Nisolle1, C. Munaut2

1CHU Liège, Obstetrics and Gynaecology, Liège, Belgium

2University of Liège, Laboratory of Tumour and Development Biology- Giga-Cancer, Liège, Belgium

3CHU Liège, Radiodiagnostic, Liège, Belgium

4UMons, CMMI - Centre de Recherche en Microscopie et Imagerie Médicale, Mons, Belgium

Background

Adenomyosis is a benign gynaecological disorder characterized by ectopic endometrial tissue within

the myometrium, leading to pain, abnormal uterine bleeding and infertility. While MRI is a widely

used diagnostic tool in human adenomyosis, its application in animal models remains underexplored.

Murine models of adenomyosis primarily rely on histological analysis for diagnosis, which is invasive

and often requires euthanasia. There is a significant gap in non-invasive, longitudinal diagnostic

methods for studying adenomyosis in mice, limiting the ability to track disease progression over time

or assess therapeutic interventions without resorting to invasive techniques.

Methods

We induced adenomyosis in mice by treating neonatal CD-1 mice with tamoxifen from days 2 to 5

after birth. Control mice received only the vehicle and did not develop adenomyosis. MRI scans were

performed once every two weeks from 8 to 24 weeks of age. MRI images were taken of anesthetized

mice placed in a horizontal animal probe using a 9.4 Tesla MRI. Anaesthesia was induced with 4%

isoflurane in a chamber before the animals were transferred to the animal imaging probe. Isoflurane

was continuously delivered via a nose cone. A pressure-sensitive pillow was placed on their back to

monitor respiration, the mice were wrapped for insulation, and the chamber of the imaging probe

was maintained at 25 °C. MRI images were compared between control and adenomyosis groups.

Histological analysis of the uteri was conducted post-sacrifice for further comparison.

Results

In the adenomyosis group, T2 hypersignal spots became visible starting at 12 weeks of age, with a

progressive worsening of the condition until 24 weeks, leading to extensive disseminated lesions.

MRI analysis of the adenomyosis-affected mice showed, compared to controls, a loss of uterine

architecture and the progressive development of increasing numbers of cystic lesions leading to

diffuse and severe adenomyosis. Histological analysis also confirmed the loss of structure and the

presence of extensive cystic lesions. These T2 hypersignal spots are similarly described for MRI-based

diagnosis of adenomyosis in humans. The results are highly promising, demonstrating that MRI can

accurately detect adenomyosis as early as 3 months of age in CD1 mice with tamoxifen-induced

adenomyosis. The findings suggest that MRI could be a reliable tool for precise, non-invasive

diagnosis of adenomyosis in murine models, with implications for future studies. One limitation of

this technique is that adenomyosis is typically diagnosed at a relatively advanced age in mice, which

may hinder early detection due to the disease’s progressive nature and the late onset of detectable

lesions.

Conclusions

MRI (9.4T) enables non-invasive detection of adenomyosis in a murine model. This novel MRI-based

diagnostic approach for murine adenomyosis facilitates longitudinal studies and real-time disease

monitoring. It enhances experimental efficiency while minimizing the need for invasive procedures

and euthanasia.

https://esge.covr.be/cmdocumentmanagement/conferencemanager/documents/api/getdocument/

14904/cmabstsms/0000201380/3fb3fb093e27fa5d1a89fa69610db53162cb02e7f619c4d4f560aa465

85dedad